MQTT Communication Protocol

AI Vision Security & EHS

Binary / HEX Packet Format for Edge Device and Web Platform Integration

| Item | Value |
| --- | --- |
| Document Type | Protocol Specification |
| Protocol Version | 2.0 Binary/HEX |
| Based On | MQTT_AI_Vision_Security_EHS_Simplified_Protocol_Updated.docx v1.1 |
| Payload Method | Raw binary bytes; shown as HEX only for documentation/debugging |
| MQTT Topic Model | Single topic per site |
| Message Types | 1 = alert_snapshot, 2 = heartbeat |
| Image Transport | Raw JPEG bytes appended after fixed metadata header |
| Header Length | 96 bytes (0x60) |
| Recommended QoS | Alert snapshot QoS 1; heartbeat QoS 0 or 1 |

Production Recommendation Use raw binary MQTT payload: [96-byte metadata header] + [raw JPEG bytes]. HEX examples in this document are only human-readable views of the actual bytes. Do not publish ASCII HEX text in production unless you intentionally select a debug mode.

# 1. Purpose

This document defines a compact binary MQTT payload for an AI Vision system used for telecom site security and EHS monitoring. It converts the earlier simple JSON + image_hex design into an RMS-style binary packet while keeping the same business meaning: alert snapshots when AI detects a security/EHS event, and heartbeat messages so the platform knows the AI Vision system is alive.

The protocol is designed for both edge-device developers and web/backend developers. The edge device builds one binary packet and publishes it on one site topic. The backend reads the fixed header, stores metadata in the database, and saves the JPEG bytes as an image file/object.

Simple Rule One topic per site. Use message_type to distinguish alert_snapshot from heartbeat. Use event_type for the main alert category and ehs_code fields for detailed EHS issues.

# 2. MQTT Topic

Each site publishes AI vision messages to one MQTT topic.

telecom/{tenant_id}/{site_id}/{device_id}/ai

| Topic Segment | Example | Notes |
| --- | --- | --- |
| tenant_id | mobiserve | Customer/operator/tenant identifier |
| site_id | ISB0167 | Telecom/site/location identifier |
| device_id | SMU02 | Gateway or Controller ID |

## Topic Example

telecom/mobiserve/ISB0167/SMU02/ai

# 3. Payload Method: Binary Bytes, HEX View

The MQTT payload is binary bytes. For documentation and testing, the same bytes can be displayed as hexadecimal values. This is the same practical method used in the RMS binary packet approach.

| Method | Use | Production Recommendation |
| --- | --- | --- |
| Raw binary bytes | Actual MQTT payload | Recommended |
| HEX view | Documentation, logs, packet examples | Allowed for debugging |
| ASCII HEX string | Text form such as "A1560101..." | Debug/commissioning only; about 2x larger |
| JSON with image_hex | Earlier simple document format | Easy but heavier; not recommended for production snapshots |

Why binary for AI Vision? AI snapshots are much larger than RMS telemetry. A 91 KB JPEG remains about 91 KB in binary, but becomes about 182 KB when converted to ASCII HEX text. Binary is better for slow telecom links, broker load, and backend processing.

# 4. Packet Model

[ Fixed 96-byte metadata header ][ Raw JPEG image bytes ]

Alert snapshot: message_type = 1, image_size_bytes > 0
Heartbeat:      message_type = 2, image_size_bytes = 0, no image_data

| Packet Type | Payload Structure | Image Included |
| --- | --- | --- |
| Alert Snapshot | 96-byte header + JPEG bytes | Yes |
| Heartbeat | 96-byte header only | No |

# 5. Encoding Rules

| Rule | Meaning | Example |
| --- | --- | --- |
| Byte order | Big-endian / network byte order | 0x0500 sent as 05 00 |
| U8 | 1-byte unsigned integer | camera_id = 2 -> 02 |
| U16 | 2-byte unsigned integer | confidence 0.910 -> 910 -> 03 8E |
| U32 | 4-byte unsigned integer | timestamp, hashes, image length |
| Confidence | U16 scaled by 1000 | 0.89 -> 890 -> 03 7A |
| Timestamps | Unix epoch UTC seconds | 2026-08-11T12:08:30Z |
| Hashes | CRC32 hash of text identifiers | site_id_hash, message_id_hash |
| Unused U8 | 0xFF where applicable | Unused EHS code slot = FF |
| Image bytes | Raw JPEG bytes appended after header | Image starts FF D8 and ends FF D9 |
| Header CRC16 | CRC16 over bytes 0x00 to 0x4F | Stored at offset 0x50 |

# 6. Fixed 96-Byte Header Mapping

The backend must decode the following fields from fixed offsets. Offset values are hexadecimal. Image data starts at offset 0x60.

| Offset | Len | Field | Type | Description |
| --- | --- | --- | --- | --- |
| 0x00 | 2 | magic | U16 | Fixed 0xA156 to identify AI Vision packet |
| 0x02 | 1 | protocol_version | U8 | 0x01 for this binary protocol version |
| 0x03 | 1 | message_type | U8 | 1=alert_snapshot, 2=heartbeat |
| 0x04 | 2 | header_length | U16 | 0x0060 = 96 bytes |
| 0x06 | 2 | flags | U16 | bit0=image_present; others reserved |
| 0x08 | 4 | packet_sequence | U32 | Incrementing sequence per edge device |
| 0x0C | 4 | timestamp_utc | U32 | Unix timestamp in UTC seconds |
| 0x10 | 4 | site_id_hash | U32 | CRC32 hash of site_id from topic |
| 0x14 | 4 | edge_device_id_hash | U32 | CRC32 hash of edge device ID |
| 0x18 | 4 | message_id_hash | U32 | CRC32 hash of unique message ID for duplicate detection |
| 0x1C | 4 | event_id_hash | U32 | CRC32 hash of event ID; 0 for heartbeat |
| 0x20 | 1 | camera_id | U8 | Camera number; 0 for heartbeat |
| 0x21 | 1 | event_type | U8 | Event enum; 0xFF for heartbeat/no event |
| 0x22 | 1 | severity | U8 | 0=info, 1=warning, 2=major, 3=critical |
| 0x23 | 2 | confidence | U16 | AI confidence x1000; 0 if heartbeat |
| 0x25 | 1 | activity_zone | U8 | Zone enum; 0=unknown/none |
| 0x26 | 2 | object_count | U16 | Number of detected persons/objects |
| 0x28 | 1 | ehs_code_count | U8 | Number of EHS codes used, 0 to 8 |
| 0x29 | 1 | ehs_code_1 | U8 | First EHS violation code; 0xFF unused |
| 0x2A | 1 | ehs_code_2 | U8 | Second EHS violation code; 0xFF unused |
| 0x2B | 1 | ehs_code_3 | U8 | Third EHS violation code; 0xFF unused |
| 0x2C | 1 | ehs_code_4 | U8 | Fourth EHS violation code; 0xFF unused |
| 0x2D | 1 | ehs_code_5 | U8 | Fifth EHS violation code; 0xFF unused |
| 0x2E | 1 | ehs_code_6 | U8 | Sixth EHS violation code; 0xFF unused |
| 0x2F | 1 | ehs_code_7 | U8 | Seventh EHS violation code; 0xFF unused |
| 0x30 | 1 | ehs_code_8 | U8 | Eighth EHS violation code; 0xFF unused |
| 0x31 | 1 | snapshot_reason_code | U8 | 0=none, 1=helmet, 2=vest, 3=multiple_ehs, 4=restricted_zone, 5=intruder, 6=crowd, 7=team_activity, 8=compliant |
| 0x32 | 1 | active_camera_count | U8 | Cameras currently online/active |
| 0x33 | 1 | configured_camera_count | U8 | Total configured cameras at the site |
| 0x34 | 1 | detection_enabled | U8 | 0=off, 1=on |
| 0x35 | 1 | system_status | U8 | Heartbeat status enum |
| 0x36 | 2 | heartbeat_interval_sec | U16 | Configured heartbeat interval, normally 60 |
| 0x38 | 4 | edge_uptime_sec | U32 | Edge device uptime in seconds |
| 0x3C | 1 | cpu_usage_percent | U8 | 0 to 100 |
| 0x3D | 1 | ram_usage_percent | U8 | 0 to 100 |
| 0x3E | 1 | disk_free_percent | U8 | 0 to 100 |
| 0x3F | 2 | camera_status_bitmap | U16 | bit0=camera1 online, bit1=camera2 online, etc. |
| 0x41 | 1 | model_id | U8 | AI model ID/version reference; 0=unknown |
| 0x42 | 1 | image_format | U8 | 0=none, 1=jpeg |
| 0x43 | 1 | image_encoding | U8 | 0=raw binary, 1=ASCII HEX debug |
| 0x44 | 2 | image_width | U16 | Pixels; 0 for heartbeat |
| 0x46 | 2 | image_height | U16 | Pixels; 0 for heartbeat |
| 0x48 | 4 | image_size_bytes | U32 | JPEG byte length; 0 for heartbeat |
| 0x4C | 4 | image_crc32 | U32 | CRC32 of JPEG bytes; 0 for heartbeat |
| 0x50 | 2 | header_crc16 | U16 | CRC16-Modbus over bytes 0x00 to 0x4F |
| 0x52 | 14 | reserved_future | bytes | Set to 00; reserved for future fields |
| 0x60 | variable | image_data | bytes | Raw JPEG bytes for alert_snapshot only |

# 7. Message Type Enum

| Value | Name | Purpose | Image |
| --- | --- | --- | --- |
| 1 | alert_snapshot | Published when AI detects security/EHS event or compliance snapshot that should be stored. | Yes |
| 2 | heartbeat | Published periodically so the platform knows the edge AI system is alive. | No |

# 8. Event Type Enum

| Value | Name | Meaning / Web Platform Action |
| --- | --- | --- |
| 0 | helmet_violation | Helmet missing, not detected, or incorrectly worn. |
| 1 | vest_violation | Safety vest missing or not detected. |
| 2 | restricted_zone_violation | Person/object detected in a restricted zone. |
| 3 | team_activity | Team activity detected in known site area such as generator, rectifier, CP, or tower zone. |
| 4 | intruder_detection | Unknown/unauthorized person detected. |
| 5 | crowd_on_site | Crowd or too many persons detected at site. |
| 6 | ehs_compliant | Person/team appears compliant. Usually info severity. |
| 7 | ehs_violation | Generic EHS violation; use EHS code slots to show exact missing/unsafe items. |
| 8 | other_alert | Reserved for future AI/security alert not covered above. |
| 255 | none | No event; used for heartbeat. |

# 9. EHS Violation Codes

For event_type = 7, use ehs_code_count and ehs_code_1 through ehs_code_8. Multiple EHS issues can be sent in the same snapshot. Unused code slots must be 0xFF.

| Code | Violation Detail |
| --- | --- |
| 0 | Helmet missing / incorrect helmet |
| 1 | Vest missing |
| 2 | Harness missing / incorrect harness |
| 3 | Chin strap missing / open |
| 4 | Safety shoes missing |
| 5 | Gloves missing |
| 6 | Y-lanyard missing / not connected |
| 7 | Positioning lanyard missing / not connected |
| 8 | Unsafe height / tower position |
| 9 | Inside restricted zone |
| 10 | Other EHS violation |
| 255 | Unused slot / not applicable |

Example Helmet + chin strap + gloves missing: ehs_code_count=3, ehs_code_1=0, ehs_code_2=3, ehs_code_3=5, remaining EHS code slots=0xFF.

# 10. Activity / Zone Enum

| Value | Zone Name | Use Case |
| --- | --- | --- |
| 0 | unknown | Zone could not be classified. |
| 1 | generator_zone | Team/person detected near generator area. |
| 2 | rectifier_zone | Team/person detected near rectifier or power cabinet. |
| 3 | cp_zone | Team/person detected near CP/control/power zone. |
| 4 | tower_zone | Team/person detected near tower/climbing area. |
| 5 | main_gate | Detected at main gate or entry point. |
| 6 | fuel_zone | Detected near fuel tank/generator fuel area. |

# 11. Severity and Heartbeat Status Enums

| Severity | Name | Recommended Portal Meaning |
| --- | --- | --- |
| 0 | info | Informational, no action required; e.g., compliant activity snapshot. |
| 1 | warning | Low-priority issue or early warning. |
| 2 | major | Action required; e.g., helmet/vest/restricted zone violation. |
| 3 | critical | Immediate action required; e.g., intruder at night or serious EHS risk. |

| system_status | Name | Meaning |
| --- | --- | --- |
| 0 | normal | Edge device alive, cameras active, detection enabled. |
| 1 | degraded | System alive but resource/camera/model issue exists. |
| 2 | camera_issue | One or more cameras offline or frame capture failed. |
| 3 | detection_off | System alive but detection disabled/stopped. |
| 4 | comm_issue | Edge device has camera/network/model communication issue. |

# 12. Alert Snapshot Packet Example

Example: camera 2 detects a generic EHS violation in tower zone. Helmet, chin strap, and gloves are missing. The following HEX is a debug view of the binary header. In the actual MQTT payload, these are bytes, followed immediately by raw JPEG bytes.

Alert snapshot debug HEX view

A1 56 01 01 00 60 00 01 00 00 00 2C 6A 7B 10 BE
00 F5 DD D8 3F F9 90 A1 35 05 24 53 51 7F C0 94
02 07 02 03 8E 04 00 01 03 00 03 05 FF FF FF FF
FF 03 02 02 01 00 00 3C 00 05 46 00 25 3D 48 00
03 01 01 00 05 00 02 D0 00 01 64 40 9B F4 8C 21
1D D9 00 00 00 00 00 00 00 00 00 00 00 00 00 00
FF D8 FF E0 00 10 4A 46 49 46 ... FF D9

| Field | Decoded Value |
| --- | --- |
| message_type | 1 = alert_snapshot |
| event_type | 7 = ehs_violation |
| severity | 2 = major |
| confidence | 910 / 1000 = 0.910 |
| camera_id | 2 |
| activity_zone | 4 = tower_zone |
| ehs_code_count | 3 |
| EHS codes | 0=helmet, 3=chin strap, 5=gloves |
| active/configured cameras | 2 / 2 |
| detection_enabled | 1 = ON |
| image_format / encoding | 1=JPEG, 0=raw binary |
| image_size_bytes | 91,200 in this example |
| image_data | Starts at offset 0x60 and begins with JPEG marker FF D8 |

# 13. Heartbeat Packet Example

Heartbeat uses the same topic and same 96-byte header. No image is appended. image_size_bytes is 0 and image_data is not present.

Heartbeat debug HEX view

A1 56 01 02 00 60 00 00 00 00 00 2D 6A 7B 10 DC
00 F5 DD D8 3F F9 90 A1 92 93 E4 6B 00 00 00 00
00 FF 00 00 00 00 00 00 00 FF FF FF FF FF FF FF
FF 00 02 02 01 00 00 3C 00 05 46 00 25 3D 48 00
03 01 01 00 00 00 00 00 00 00 00 00 00 00 00 00
CD 08 00 00 00 00 00 00 00 00 00 00 00 00 00 00

| Field | Decoded Value |
| --- | --- |
| message_type | 2 = heartbeat |
| event_type | 255 = none |
| active_camera_count | 2 |
| configured_camera_count | 2 |
| detection_enabled | 1 = ON |
| system_status | 0 = normal |
| heartbeat_interval_sec | 60 |
| edge_uptime_sec | 345600 |
| cpu / ram / disk | 37% / 61% / 72% |
| camera_status_bitmap | 0x0003 = camera 1 and 2 online |
| image_size_bytes | 0 |

# 14. Edge Device Development Rules

Publish all AI Vision messages to the single site topic: aivision/{tenant_id}/{site_id}/vision.

Send message_type=1 immediately when a relevant violation, security alert, crowd event, team activity, or EHS compliance snapshot is detected.

Send message_type=2 heartbeat every 60 seconds by default. Do not append image bytes to heartbeat.

Compress snapshots as JPEG before publishing. Target JPEG size should normally be 50 KB to 150 KB before adding the header.

Build the 96-byte binary header, calculate image_crc32 and header_crc16, then append raw JPEG bytes.

Use packet_sequence and message_id_hash/event_id_hash so the backend can ignore duplicate MQTT retries.

Use UTC timestamp from the edge device. If time is not synchronized, still send the message and let the backend mark timestamp quality from device configuration/logs.

When multiple EHS rules fail in the same frame, send one event_type=7 packet with ehs_code_count and multiple ehs_code slots.

# 15. Backend/Web Platform Decode Rules

Subscribe to aivision/+/+/vision or tenant-specific topic filters as required.

Read the first 96 bytes as the fixed header. Reject packets with wrong magic, unsupported protocol_version, or invalid header_length.

Validate header_crc16 before trusting metadata.

For message_type=1, validate event_type, confidence, camera_id, image_size_bytes, and image_crc32. Save image_data as a JPEG file/object.

Use event_type for the main dashboard alert category. Use ehs_code_count and ehs_code_1..8 for detailed EHS labels.

Use message_id_hash/event_id_hash plus packet_sequence for de-duplication, especially with MQTT QoS 1 retries.

For message_type=2, update live AI edge status, camera status, detection_enabled, and last heartbeat time.

Mark the AI Vision system offline if no heartbeat is received for 3 heartbeat intervals.

Backend decode example

// Node.js backend decode example
const HEADER_SIZE = 96;

function decodeAiVisionPacket(payload) {
  const magic = payload.readUInt16BE(0x00);
  if (magic !== 0xA156) throw new Error('Invalid AI Vision packet magic');

  const version = payload.readUInt8(0x02);
  const messageType = payload.readUInt8(0x03);
  const headerLength = payload.readUInt16BE(0x04);
  const packetSequence = payload.readUInt32BE(0x08);
  const timestampUtc = payload.readUInt32BE(0x0C);
  const cameraId = payload.readUInt8(0x20);
  const eventType = payload.readUInt8(0x21);
  const severity = payload.readUInt8(0x22);
  const confidence = payload.readUInt16BE(0x23) / 1000;
  const activityZone = payload.readUInt8(0x25);
  const ehsCodeCount = payload.readUInt8(0x28);
  const ehsCodes = [];
  for (let i = 0; i < Math.min(ehsCodeCount, 8); i++) {
    const code = payload.readUInt8(0x29 + i);
    if (code !== 0xFF) ehsCodes.push(code);
  }

  const imageSize = payload.readUInt32BE(0x48);
  const imageBytes = payload.slice(headerLength, headerLength + imageSize);

  return { messageType, packetSequence, timestampUtc, cameraId, eventType,
           severity, confidence, activityZone, ehsCodes, imageBytes };
}

# 16. Suggested Database Handling

| Table | Purpose | Main Fields |
| --- | --- | --- |
| ai_vision_events | Stores alert_snapshot metadata | event_id_hash, site_id, camera_id, event_type, severity, confidence, ehs_codes, image_path, timestamp_utc |
| ai_vision_devices | Stores latest heartbeat per edge device | edge_device_id, site_id, last_heartbeat_at, system_status, active_camera_count, detection_enabled |
| ai_vision_camera_status | Stores latest camera status | site_id, edge_device_id, camera_id, online, last_frame_age_sec or status timestamp |

# 17. Portal Status Logic

| Condition | Portal Status |
| --- | --- |
| Last heartbeat age <= 3 x heartbeat_interval_sec and system_status = 0 | Online / Normal |
| Last heartbeat age <= 3 x heartbeat_interval_sec and system_status = 1 | Online / Degraded |
| One or more cameras offline or active_camera_count < configured_camera_count | Camera Issue |
| detection_enabled = 0 | Detection Off |
| No heartbeat for more than 3 intervals | AI Vision Offline |
| New alert_snapshot received with severity 2 or 3 | Active Alert / Action Required |

# 18. Production Defaults

| Item | Recommended Setting |
| --- | --- |
| Alert snapshot QoS | QoS 1 |
| Heartbeat QoS | QoS 0 for high scale, QoS 1 for smaller/private deployments |
| Retain flag | False |
| Heartbeat interval | 60 seconds |
| Offline threshold | 3 missed heartbeats |
| Image format | JPEG |
| Image encoding | Raw binary bytes appended after header |
| Target JPEG size | 50 KB to 150 KB before adding header |
| Maximum recommended MQTT payload | Keep below 500 KB where possible on slow links |
| Transport security | TLS MQTT with username/password or certificate authentication |

# 19. Final Summary

Recommended Working Design Use one site topic and two binary message types. Alert snapshots carry a 96-byte metadata header plus raw JPEG bytes. Heartbeats carry only the 96-byte header. The backend identifies the message from message_type, maps event_type to the alert category, maps ehs_code slots to EHS details, saves image bytes as JPEG, and uses heartbeat to determine whether the AI Vision system is alive.